SECTION 11 68 33.13

FOOTBALL FIELD EQUIPMENT

PART 1 – GENERAL

- SUMMARY

  - Section Includes: Jaypro FBSLV8 Standard Football Goal Post Ground Sleeve for 6-5/8 in. O.D. Goosenecks

  - Related Sections

    - Section 03 30 00 – Cast-In-Place Concrete

    - Section 31 23 13 – Subgrade Preparation

    - Section 31 23 16 – Excavation

    - Section 31 23 23.13 – Backfill

    - Section 32 12 16.36 – Athletic Asphalt Paving

    - Section 32 18 23.29 – Synthetic Field Sport Surfacing

    - Section 32 18 23.39 – Synthetic Running Track Surfacing

- REFERENCES

  - Abbreviations and Acronyms

    - National Federation of State High School Associations (NFHS)

      - Provides standards for high school competition.

    - National Collegiate Athletic Association (NCAA)

      - Provides standards for collegiate competition.

    - Outside diameter shall be expressed as O.D.

    - Measurements in feet shall be expressed as ft.

    - Measurements in inches shall be expressed as in.

  - Reference Standards: Jaypro Sports LLC, along with the NCAA and the NFHS, are the standards for quality required and specified herein.

- SUBMITTALS

  - Comply with Section 01 33 00 – Submittal Procedures

    - Product Data: Submit manufacturer’s product literature, technical specifications, including materials, components, fabrication, and finish, prior to actual field installation work for Architect or Owner’s Representative review.

    - Shop Drawings

    - Manufacturer’s Instructions: Provide drawings of manufacturer’s recommended installation and foundation requirements prior to actual field installation work for Architect or Owner’s Representative review.

    - Warranty Documentation: Manufacturer’s warranties shall pass to the Owner and certification made that the product materials meet all applicable grade trademarks or conform to industry standards and inspection requirements.

- QUALITY ASSURANCE

  - Installer Qualifications: Contractor experienced with track & field installation, excavation, concrete work, asphalt paving and track surfacing.

- DELIVERY, STORAGE, AND HANDLING

  - Delivery: Deliver all components grouped together in original manufacturer’s packaging with labels clearly labeling manufacturer and contents. Materials delivered to the site shall be examined immediately for concealed damage or defects in shipping. Any defects shall be noted and reported to the Owner’s Representative.

## Storage: Sound materials shall be stored above the ground under protective cover, or indoors, to provide proper protection.

## Handling: Handle all components in a fashion that protects and preserves all components and finishes during delivery, storage, and installation.

## Replacements: If necessary, replacements shall be immediately re-ordered to minimize any conflict with the construction schedule.

PART 2 – PRODUCTS

- MANUFACTURER

  - Jaypro Sports, LLC, 976 Hartford Tpk, Waterford, CT 06385. Toll Free (800) 243-0533, Phone (860) 447-3001, Fax (800)988-3363 www.jaypro.com

  - Substitution Limitations

    - Only with the approval of the architect will substitutions for materials specified, be allowed; see Section 01 25 00 – Substitution Procedures.

    - The work has been designed and detailed based upon the first specified manufacturer’s materials equipment, products, sizes, colors, etc. Contractors are advised that any and all modifications to be contractor’s scope of work or to new or existing work resulting from the contractor’s approved substitution request shall be the sole responsibility of the contractor and shall be performed by the contractor at NO additional cost to the client/district.

    - Contractor shall notify architect, prior to bid time, of any items which are obsolete or will not be available for use in this project.

    - If the contractor’s approved substitution request is a different size than the specified item, contractor shall adjust all supporting and surrounding construction as directed by the architect and as required for the different sized item, all at NO additional cost to the client/district.

- SYSTEM DESCRIPTION

  - This ground sleeve is intended for use with the Max-1, 6-5/8 in. O.D. Football Goal Posts. Ground sleeve shall be constructed of an 8.625 in. O.D. x 0.322 in. wall x 60 in. long steel pipe with a 10 in. x 10 in. x 1/4 in. thick steel plate welded to end for placement on concrete. Top of ground sleeve shall have two 11/16 in. holes, 180 degrees apart, for anti-rotation rods. Below these holes, there shall be three 3/4 in. holes, 120 degrees apart, with 1/2 in. – 13 square weld nuts welded at them for alignment bolts.

- SYSTEM FINISH

  - Ground sleeve and hardware shall be galvanized or zinc primed.

- ACCESSORIES

  - Access Frame and Covers (Jaypro Model FGBPAF-1)

PART 3 – EXECUTION

- EXAMINATION

  - Examine the areas and conditions where equipment and systems are to be installed and notify the contractor of conditions detrimental to the proper and timely installation of the work.

  - Do not proceed with the work until unsatisfactory conditions have been corrected by the contractor in a manner acceptable and to the satisfaction of the Architect or Owner’s Representative.

- INSTALLATION

  - All athletic equipment shall be installed as indicated on approved submittals as recommended and in strict accordance with manufacturer's written directions and as indicated on the drawings and specified herein.

  - All concrete footings for athletic equipment shall be installed as indicated on the drawings and in accordance with Section 03 30 00 - Cast-in-Place Concrete.

  - All sleeves required for athletic equipment installation shall be set plumb and true to line and grade in concrete as indicated on the drawings and per manufacturer's recommendation.

  - All athletic equipment shall be installed in strict accordance with the latest rules, regulations and specifications governing that sport or event for which it is being installed.

  - Remove and replace any damaged components that cannot be successfully repaired.

- ADJUSTING

  - All athletic equipment requiring testing, adjustment and operation shall be tested for proper operation and adjusted to conform to specified standards.

  - Turnover operating and maintenance instructions and manuals to Owner designated personnel for proper operation and care of equipment.

- CLEANING

  - Upon completion of work, clean the equipment free of dirt, mud or stains. Touch up scratches with matching paint.

  - Do not use harsh cleaning materials or methods that would damage the finish.

  - Do not remove warning labels or decals.

  - Completely clean-up work area.  Remove all trash and debris.

END OF SECTION