SECTION 11 66 23.23

VOLLEYBALL EQUIPMENT

PART 1 – GENERAL

- SECTION INCLUDES

  - Jaypro PVBN-6AK Flex Net International with Adapter Cords

- REFERENCES

  - Abbreviations and Acronyms

    - National Federation of State High School Associations (NFHS)

      - Provides standards for high school competition.

    - National Collegiate Athletic Association (NCAA)

      - Provides standards for collegiate competition.

    - United States Volleyball Association (USVBA)

      - Provides standards for volleyball regulations.

    - Outside diameter shall be expressed as O.D.

    - Inside diameter shall be expressed as I.D.

    - Measurements in feet shall be expressed as ft.

    - Measurements in inches shall be expressed as in.

  - Reference Standards: Jaypro Sports LLC, along with the NFHS, NCAA, and the USVBA, are the standards for quality required and specified herein.

- SUBMITTALS

  - Comply with Section 01 33 00 – Submittal Procedures

    - Product Data: Submit manufacturer’s product literature, technical specifications, including materials, components, fabrication, and finish, prior to actual field installation work for Architect or Owner’s Representative review.

    - Shop Drawings

    - Manufacturer’s Instructions: Provide drawings of manufacturer’s recommended installation and foundation requirements prior to actual field installation work for Architect or Owner’s Representative review.

    - Warranty Documentation: Manufacturer’s warranties shall pass to the Owner and certification made that the product materials meet all applicable grade trademarks or conform to industry standards and inspection requirements.

- DELIVERY, STORAGE, AND HANDLING

## Delivery: Deliver all components grouped together in original manufacturer’s packaging with labels clearly labeling manufacturer and contents. Materials delivered to the site shall be examined immediately for concealed damage or defects in shipping. Any defects shall be noted and reported to the Owner’s Representative.

## Storage: Sound materials shall be stored above the ground under protective cover, or indoors, to provide proper protection.

## Handling: Handle all components in a fashion that protects and preserves all components and finishes during delivery, storage, and installation.

## Replacements: If necessary, replacements shall be immediately re-ordered to minimize any conflict with the construction schedule.

PART 2 – PRODUCTS

- MANUFACTURER

  - Jaypro Sports, LLC, 976 Hartford Tpk, Waterford, CT 06385. Toll Free (800) 243-0533, Phone (860) 447-3001, Fax (800)988-3363 www.jaypro.com

  - Substitution Limitations

    - Only with the approval of the architect will substitutions for materials specified, be allowed; see Section 01 25 00 – Substitution Procedures.

    - The work has been designed and detailed based upon the first specified manufacturer’s materials equipment, products, sizes, colors, etc. Contractors are advised that any and all modifications to be contractor’s scope of work or to new or existing work resulting from the contractor’s approved substitution request shall be the sole responsibility of the contractor and shall be performed by the contractor at NO additional cost to the client/district.

    - Contractor shall notify architect, prior to bid time, of any items which are obsolete or will not be available for use in this project.

    - If the contractor’s approved substitution request is a different size than the specified item, contractor shall adjust all supporting and surrounding construction as directed by the architect and as required for the different sized item, all at NO additional cost to the client/district.

- SYSTEM DESCRIPTION

  - Designed for International, Collegiate and High School competition. The Patented Flex Net eliminates all cumbersome cables and replaces them with heavy duty, woven nylon, top and bottom bands. The headband attaches directly to the winch leader strap to provide superb net tension.  Patent # 5,358,257. The adapter cord allows for connection of existing, non-Jaypro, winches and uprights to the flex net.

- SYSTEM MATERIALS

  - Body: Net shall be 32 ft. long x 39 in. high with a top cable-less binding of 32 ft. 6 in.. A 2 in., 10,000 lbs. tensile strength, white binding shall surround entire perimeter of net body. High tensile strength hardened steel delta rings are attached to allow connection to upright leader straps or cables. Bottom of net shall have, on each end, a 2 in. Velcro tensioning strap and metal buckle to allow a very tight bottom net binding. Each buckle shall be covered with an attached, yet removable, Velcro strap for player safety. Standard overall net length is designed for 36 ft. on-center standards, but will accommodate 35 ft. to 37 ft. spacing. Volleyball nets with cables for tensioning shall not be considered equal.

  - Netting: Shall be manufactured from high quality, #21 black thread knotted nylon, 4 in. square mesh. Two net dowels at each end shall consist of 1/2 in. EMT steel tubing which are completely enclosed for safety in side pockets.

  - Adapter Cord: Shall be constructed of a 1/4 in. O.D. cord with clips for connection from winch and anchor to top of net.

- ACCESSORIES

  - Custom Volleyball Net Tape (Jaypro Model CVNT-1)

  - Single, Double or Triple Net Keeper (Jaypro Model VNK11, VNK22, VNK33)

  - Replacement Net Keeper Storage Bag (Jaypro Model  VNKBAG)

  - Cable/Buckle Cover (Jaypro Model VCC-12)

  - Boundary Tapes (Jaypro Model VA-22)

PART 3 – EXECUTION

- EXAMINATION

  - Examine the areas and conditions where equipment and systems are to be installed and notify the contractor of conditions detrimental to the proper and timely installation of the work.

  - Do not proceed with the work until unsatisfactory conditions have been corrected by the contractor in a manner acceptable and to the satisfaction of the Architect or Owner’s Representative

- INSTALLATION

  - All athletic equipment shall be installed as indicated on approved submittals as recommended and in strict accordance with manufacturer's written directions and as indicated on the drawings and specified herein.

  - All athletic equipment shall be installed in strict accordance with the latest rules, regulations and specifications governing that sport or event for which it is being installed.

  - Remove and replace any damaged components that cannot be successfully repaired.

- ADJUSTING

  - All athletic equipment requiring testing, adjustment and operation shall be tested for proper operation and adjusted to conform to specified standards.

  - Turnover operating and maintenance instructions and manuals to Owner designated personnel for proper operation and care of equipment.

- CLEANING

  - Upon completion of work, clean the equipment free of dirt, mud or stains. Touch up scratches with matching paint.

  - Do not use harsh cleaning materials or methods that would damage the finish.

  - Do not remove warning labels or decals.

  - Completely clean-up work area. Remove all trash and debris.

END OF SECTION